Lab Validation Runbook - Magnetic Circuit, Rev A

Synthetic hackathon engineering evidence

# 1. Wiring and references

Connect the exciting coil with the documented polarity dot orientation.

Connect the measuring coil so its recorded induced voltage is negative relative to the positive air-gap flux arrow.

Confirm electrical ground and magnetic ground are instantiated in the simulation model.

Use the nominal 1.50 mm gap for AV-11. The 1.58 mm shim value belongs to the as-built prototype comparison only.

# 2. Excitation

Apply 50 Hz quasi-static excitation with an RMS current-phasor magnitude ramp: 0 A until 0.10 s, linear ramp to 2.0 A by 0.50 s, then hold to 1.0 s.

# 3. Expected trends

| Signal | Expected behavior |
| --- | --- |
| N*I | Linear ramp with current |
| Phi_core | Linear ramp under linear reluctance |
| Phi_gap | 0.92*Phi_core |
| Phi_leak | 0.08*Phi_core |
| B and H | Linear ramp |
| Exciting induced-voltage magnitude | Linear ramp; positive orientation |
| Measuring induced-voltage magnitude | Linear ramp; negative orientation |

# 4. Common errors

Using 2.0 A as a peak rather than RMS phasor magnitude causes a sqrt(2) mismatch.

Using the as-built 1.58 mm gap in the nominal verification changes the flux and invalidates the AV-11 comparison.

Applying total core flux to the air gap instead of useful flux ignores leakage and overpredicts air-gap B.

Omitting one of the grounds can leave the corresponding potential reference undefined.